Table of Contents

Building a Camera-Based Human Pose Estimation System for Barbell Velocity Tracking	3

A Comprehensive Technical Guide for Velocity-Based Training Applications	3

Executive Summary	4

Table of Contents	4

1. Introduction and Background	4

1.1 What is Velocity-Based Training?	4

1.2 Why Camera-Based Tracking?	5

1.3 System Goal	5

2. System Architecture Overview	5

2.1 High-Level Pipeline	5

2.2 Core Components	5

2.3 Data Flow	6

3. Hardware Requirements	6

3.1 Camera Specifications	6

Minimum Requirements	6

Frame Rate Considerations	6

Recommended Camera Options	6

3.2 Computing Hardware	7

CPU Requirements	7

GPU Acceleration	7

Memory Requirements	7

3.3 Additional Equipment	7

4. Software Stack and Dependencies	7

4.1 Programming Language	7

4.2 Core Dependencies	8

4.3 Installation	8

5. Pose Estimation Model Selection	8

5.1 Available Models Comparison	8

5.2 Recommended: MediaPipe Pose	8

Key Features	8

Relevant Landmarks for Squat Tracking	9

5.3 MediaPipe Configuration	9

Model Complexity Trade-offs	9

6. Camera Setup and Calibration	9

6.1 Camera Positioning	9

Optimal Setup	9

Why Sagittal Plane View?	10

Setup Diagram	10

6.2 Spatial Calibration	10

Method 1: Known Object Reference	10

Method 2: Checkerboard Calibration	10

6.3 Temporal Calibration	11

7. Joint Position Extraction	12

7.1 MediaPipe Landmark Extraction	12

7.2 Tracking Point Selection	14

Option 1: Hip Center (Recommended)	14

Option 2: Shoulder Center	14

Option 3: Direct Barbell Detection	14

7.3 Handling Missing Detections	15

8. Velocity Calculation Algorithms	16

8.1 Basic Velocity Calculation	16

8.2 Multi-Point Velocity Estimation	16

8.3 Phase Detection	17

8.4 Key Metrics Calculation	18

9. Signal Processing and Filtering	19

9.1 Why Filtering is Necessary	19

9.2 Low-Pass Butterworth Filter	20

Cutoff Frequency Selection	20

9.3 Savitzky-Golay Filter	21

9.4 Filter Comparison and Selection	21

9.5 Real-Time vs. Post-Processing Considerations	22

10. Real-Time Implementation	23

10.1 System Architecture for Real-Time	23

10.2 Display and Feedback	26

10.3 Performance Optimization	27

11. Validation and Accuracy Assessment	27

11.1 Validation Against Gold Standard	27

11.2 Validation Protocol	28

11.3 Bland-Altman Analysis	29

11.4 Expected Accuracy	30

12. Practical Considerations and Limitations	30

12.1 Environmental Factors	30

Lighting	30

Background	30

Clothing	30

12.2 Movement Considerations	30

Occlusion	30

Lateral Movement	31

Bar Whip	31

12.3 System Limitations	31

12.4 When NOT to Use Camera-Based Tracking	31

13. Sample Implementation Code	31

13.1 Complete Basic Implementation	31

14. Troubleshooting Guide	39

14.1 Common Issues and Solutions	39

14.2 Diagnostic Checks	39

15. References and Further Reading	40

15.1 Academic References	40

15.2 Technical Documentation	41

15.3 VBT Resources	41

Appendix A: Quick Start Checklist	41

Appendix B: Glossary	41

# Building a Camera-Based Human Pose Estimation System for Barbell Velocity Tracking

## A Comprehensive Technical Guide for Velocity-Based Training Applications

# Executive Summary

This document provides a complete technical guide for building a camera-based velocity tracking system using Human Pose Estimation (HPE) to measure barbell velocity during back squats. This system serves as a cost-effective alternative to commercial Velocity-Based Training (VBT) devices, leveraging modern computer vision and machine learning techniques.

The guide covers hardware requirements, software architecture, pose estimation model selection, velocity calculation algorithms, calibration procedures, and validation methods to achieve measurement accuracy within ±0.05 m/s of commercial linear position transducers.

# Table of Contents

- Introduction and Background

- System Architecture Overview

- Hardware Requirements

- Software Stack and Dependencies

- Pose Estimation Model Selection

- Camera Setup and Calibration

- Joint Position Extraction

- Velocity Calculation Algorithms

- Signal Processing and Filtering

- Real-Time Implementation

- Validation and Accuracy Assessment

- Practical Considerations and Limitations

- Sample Implementation Code

- Troubleshooting Guide

- References and Further Reading

# 1. Introduction and Background

## 1.1 What is Velocity-Based Training?

Velocity-Based Training (VBT) is a training methodology that uses movement velocity as the primary metric for prescribing and monitoring resistance training loads. Rather than relying solely on percentage-based programming (e.g., 80% of 1RM), VBT allows athletes and coaches to:

- Autoregulate training loads based on daily readiness

- Monitor fatigue accumulation within and across sessions

- Establish load-velocity profiles for accurate 1RM estimation

- Optimize power output by targeting specific velocity zones

- Reduce injury risk by avoiding excessive fatigue

## 1.2 Why Camera-Based Tracking?

Commercial VBT devices (GymAware, PUSH Band, Tendo Unit, etc.) range from $150 to over $2,000. A camera-based HPE system offers:

- Cost reduction: Uses existing smartphones or webcams

- No attachment required: Non-contact measurement

- Additional data: Full body kinematics, not just barbell position

- Scalability: Multiple athletes tracked simultaneously

- Research applications: Access to raw joint position data

## 1.3 System Goal

The primary objective is to track the vertical velocity of the barbell during the concentric phase of a back squat with sufficient accuracy (±0.05 m/s) for practical VBT applications including:

- Mean concentric velocity (MCV)

- Peak concentric velocity (PCV)

- Velocity at specific portions of the lift

- Rep-to-rep velocity loss for fatigue monitoring

# 2. System Architecture Overview

## 2.1 High-Level Pipeline

The velocity tracking system follows this processing pipeline:

Camera Input → Frame Capture → Pose Estimation → Joint Extraction →
Position Tracking → Velocity Calculation → Filtering → Output Display

## 2.2 Core Components

| Component | Function |
| --- | --- |
| Camera System | Captures video frames at consistent frame rate |
| Pose Estimation Model | Detects body keypoints in each frame |
| Calibration Module | Converts pixel coordinates to real-world distances |
| Tracking Algorithm | Maintains joint identity across frames |
| Velocity Calculator | Computes instantaneous and mean velocities |
| Signal Processor | Applies filters to reduce noise |
| Output Interface | Displays results and provides feedback |

## 2.3 Data Flow

Each frame undergoes the following processing:

- Frame acquisition with precise timestamp

- Pose estimation inference (typically 10-50ms)

- Keypoint extraction and confidence filtering

- Coordinate transformation (pixels to meters)

- Position differencing for velocity

- Low-pass filtering for noise reduction

- Phase detection (eccentric/concentric)

- Metric calculation and storage

# 3. Hardware Requirements

## 3.1 Camera Specifications

### Minimum Requirements

| Parameter | Minimum | Recommended | Optimal |
| --- | --- | --- | --- |
| Frame Rate | 30 FPS | 60 FPS | 120+ FPS |
| Resolution | 720p | 1080p | 1080p |
| Shutter Type | Rolling | Global | Global |
| Lens | Fixed | Wide-angle | Wide-angle |

### Frame Rate Considerations

At 30 FPS, frame interval = 33.3ms. For a typical squat concentric phase of 0.5-1.5 seconds:

- 30 FPS: 15-45 frames captured

- 60 FPS: 30-90 frames captured

- 120 FPS: 60-180 frames captured

Higher frame rates provide more data points, enabling better velocity resolution and more effective filtering.

### Recommended Camera Options

Budget ($0-50): - Existing smartphone (most modern phones support 60 FPS at 1080p) - Logitech C920/C922 webcam (30 FPS, adequate for basic use)

Mid-Range ($50-200): - Logitech BRIO (60 FPS at 1080p) - GoPro Hero (120 FPS at 1080p) - iPhone/Android with slow-motion mode

Professional ($200+): - Basler or FLIR industrial cameras (120+ FPS) - Sony RX0 II (up to 1000 FPS)

## 3.2 Computing Hardware

### CPU Requirements

| Task | Minimum | Recommended |
| --- | --- | --- |
| Real-time 30 FPS | Intel i5 / Ryzen 5 | Intel i7 / Ryzen 7 |
| Real-time 60 FPS | Intel i7 / Ryzen 7 | Intel i9 / Ryzen 9 |
| Post-processing | Any modern CPU | Multi-core preferred |

### GPU Acceleration

For real-time performance, GPU acceleration is highly recommended:

| GPU | Approximate Inference Time (MediaPipe) |
| --- | --- |
| Integrated Graphics | 50-100ms per frame |
| NVIDIA GTX 1060 | 15-25ms per frame |
| NVIDIA RTX 3070 | 5-10ms per frame |
| Apple M1/M2 Neural Engine | 8-15ms per frame |

### Memory Requirements

- Minimum: 8 GB RAM

- Recommended: 16 GB RAM (allows for video buffering and batch processing)

## 3.3 Additional Equipment

- Tripod or mounting system: Essential for consistent camera positioning

- Calibration object: Measured reference (e.g., barbell length, calibration board)

- Lighting: Consistent, even illumination (avoid backlighting)

- Display/feedback device: Screen visible to athlete for real-time feedback

# 4. Software Stack and Dependencies

## 4.1 Programming Language

Python is recommended due to: - Extensive computer vision library support - Rapid prototyping capability - Strong machine learning ecosystem - Cross-platform compatibility

## 4.2 Core Dependencies

# Core libraries
numpy>=1.21.0          # Numerical operations
opencv-python>=4.5.0   # Video capture and processing
scipy>=1.7.0           # Signal processing and filtering

# Pose estimation (choose one)
mediapipe>=0.8.10      # Google's MediaPipe Pose
# OR
torch>=1.9.0           # For OpenPose or other PyTorch models
torchvision>=0.10.0

# Optional but recommended
matplotlib>=3.4.0      # Visualization
pandas>=1.3.0          # Data management

## 4.3 Installation

# Create virtual environment
python -m venv vbt_env
source vbt_env/bin/activate  # Linux/Mac
# vbt_env\Scripts\activate   # Windows

# Install dependencies
pip install numpy opencv-python scipy mediapipe matplotlib pandas

# 5. Pose Estimation Model Selection

## 5.1 Available Models Comparison

| Model | Speed | Accuracy | Ease of Use | Best For |
| --- | --- | --- | --- | --- |
| MediaPipe Pose | Very Fast | Good | Excellent | Real-time, mobile |
| MoveNet | Fast | Good | Good | Real-time, edge devices |
| OpenPose | Slow | Excellent | Moderate | Research, multi-person |
| AlphaPose | Moderate | Excellent | Moderate | High accuracy needs |
| HRNet | Slow | Best | Complex | Maximum accuracy |

## 5.2 Recommended: MediaPipe Pose

For barbell velocity tracking, MediaPipe Pose offers the best balance of speed, accuracy, and ease of implementation.

### Key Features

- 33 body landmarks including hip, knee, and ankle

- Real-time performance on CPU (30+ FPS)

- Built-in tracking across frames

- Confidence scores for each landmark

- Cross-platform support (Python, JavaScript, mobile)

### Relevant Landmarks for Squat Tracking

| Landmark Index | Name | Use Case |
| --- | --- | --- |
| 11, 12 | Left/Right Shoulder | Upper body reference |
| 23, 24 | Left/Right Hip | Primary tracking point |
| 25, 26 | Left/Right Knee | Squat depth reference |
| 27, 28 | Left/Right Ankle | Ground reference |

## 5.3 MediaPipe Configuration

import mediapipe as mp

mp_pose = mp.solutions.pose

pose = mp_pose.Pose(
    static_image_mode=False,      # Video mode for tracking
    model_complexity=2,            # 0=Lite, 1=Full, 2=Heavy
    smooth_landmarks=True,         # Temporal smoothing
    enable_segmentation=False,     # Not needed for velocity
    min_detection_confidence=0.5,
    min_tracking_confidence=0.5
)

### Model Complexity Trade-offs

| Complexity | Inference Time | Accuracy | Recommended Use |
| --- | --- | --- | --- |
| 0 (Lite) | ~5ms | Lower | Mobile, high FPS |
| 1 (Full) | ~15ms | Good | Standard use |
| 2 (Heavy) | ~30ms | Best | Maximum accuracy |

# 6. Camera Setup and Calibration

## 6.1 Camera Positioning

### Optimal Setup

- Position: Perpendicular to the sagittal plane (side view of lifter)

- Distance: 3-5 meters from the lifter

- Height: Hip height of the lifter (approximately 0.9-1.1m)

- Angle: Camera sensor parallel to the floor

### Why Sagittal Plane View?

The primary movement in a squat is vertical. A side-on view: - Maximizes vertical displacement in the image plane - Minimizes perspective distortion for vertical measurements - Clearly shows hip and shoulder positions - Avoids occlusion from the barbell

### Setup Diagram

[Camera]
           |
           | 3-5m
           |
    ←------+------→
           |
       [Lifter]
        Squat

## 6.2 Spatial Calibration

Converting pixel coordinates to real-world distances requires calibration.

### Method 1: Known Object Reference

Use an object of known length visible in the frame:

- Measure barbell length (standard Olympic bar = 2.2m)

- In the calibration frame, identify barbell endpoints

- Calculate pixels per meter: scale = pixel_distance / real_distance

def calculate_scale(point1, point2, known_distance_meters):
    """
    Calculate pixels per meter using two points and known distance.
    
    Args:
        point1: (x, y) pixel coordinates of first point
        point2: (x, y) pixel coordinates of second point
        known_distance_meters: Real-world distance between points
    
    Returns:
        pixels_per_meter: Conversion factor
    """
    pixel_distance = np.sqrt(
        (point2[0] - point1[0])**2 + 
        (point2[1] - point1[1])**2
    )
    pixels_per_meter = pixel_distance / known_distance_meters
    return pixels_per_meter

### Method 2: Checkerboard Calibration

For higher accuracy, use OpenCV’s camera calibration:

import cv2
import numpy as np

def calibrate_camera(checkerboard_images, pattern_size=(9, 6), square_size=0.025):
    """
    Full camera calibration using checkerboard pattern.
    
    Args:
        checkerboard_images: List of image paths
        pattern_size: Internal corners (columns, rows)
        square_size: Size of each square in meters
    
    Returns:
        camera_matrix, distortion_coeffs
    """
    objp = np.zeros((pattern_size[0] * pattern_size[1], 3), np.float32)
    objp[:, :2] = np.mgrid[0:pattern_size[0], 0:pattern_size[1]].T.reshape(-1, 2)
    objp *= square_size
    
    objpoints = []
    imgpoints = []
    
    for img_path in checkerboard_images:
        img = cv2.imread(img_path)
        gray = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)
        
        ret, corners = cv2.findChessboardCorners(gray, pattern_size, None)
        
        if ret:
            objpoints.append(objp)
            corners2 = cv2.cornerSubPix(
                gray, corners, (11, 11), (-1, -1),
                (cv2.TERM_CRITERIA_EPS + cv2.TERM_CRITERIA_MAX_ITER, 30, 0.001)
            )
            imgpoints.append(corners2)
    
    ret, mtx, dist, rvecs, tvecs = cv2.calibrateCamera(
        objpoints, imgpoints, gray.shape[::-1], None, None
    )
    
    return mtx, dist

## 6.3 Temporal Calibration

Accurate velocity requires precise timing:

import time

class FrameTimer:
    def __init__(self):
        self.timestamps = []
        self.frame_count = 0
    
    def capture_timestamp(self):
        """Record timestamp for current frame."""
        self.timestamps.append(time.perf_counter())
        self.frame_count += 1
    
    def get_dt(self, frame_idx):
        """Get time difference between frames."""
        if frame_idx < 1:
            return None
        return self.timestamps[frame_idx] - self.timestamps[frame_idx - 1]
    
    def get_actual_fps(self):
        """Calculate actual achieved frame rate."""
        if len(self.timestamps) < 2:
            return None
        total_time = self.timestamps[-1] - self.timestamps[0]
        return (len(self.timestamps) - 1) / total_time

# 7. Joint Position Extraction

## 7.1 MediaPipe Landmark Extraction

import cv2
import mediapipe as mp
import numpy as np

class PoseExtractor:
    def __init__(self, model_complexity=2):
        self.mp_pose = mp.solutions.pose
        self.pose = self.mp_pose.Pose(
            static_image_mode=False,
            model_complexity=model_complexity,
            smooth_landmarks=True,
            min_detection_confidence=0.5,
            min_tracking_confidence=0.5
        )
        
        # Landmark indices
        self.LEFT_HIP = 23
        self.RIGHT_HIP = 24
        self.LEFT_SHOULDER = 11
        self.RIGHT_SHOULDER = 12
    
    def process_frame(self, frame):
        """
        Extract pose landmarks from a frame.
        
        Args:
            frame: BGR image (numpy array)
        
        Returns:
            landmarks: List of landmark positions or None
            confidence: Overall detection confidence
        """
        # Convert BGR to RGB
        rgb_frame = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)
        
        # Process frame
        results = self.pose.process(rgb_frame)
        
        if results.pose_landmarks is None:
            return None, 0.0
        
        landmarks = results.pose_landmarks.landmark
        
        # Calculate average confidence of key landmarks
        key_indices = [self.LEFT_HIP, self.RIGHT_HIP, 
                       self.LEFT_SHOULDER, self.RIGHT_SHOULDER]
        confidence = np.mean([landmarks[i].visibility for i in key_indices])
        
        return landmarks, confidence
    
    def get_hip_center(self, landmarks, frame_width, frame_height):
        """
        Calculate center point between hips in pixel coordinates.
        
        The hip center is the recommended tracking point for barbell
        position as it closely approximates barbell vertical position
        in a back squat.
        """
        left_hip = landmarks[self.LEFT_HIP]
        right_hip = landmarks[self.RIGHT_HIP]
        
        # Convert normalized coordinates to pixels
        x = (left_hip.x + right_hip.x) / 2 * frame_width
        y = (left_hip.y + right_hip.y) / 2 * frame_height
        
        # Average visibility as confidence
        confidence = (left_hip.visibility + right_hip.visibility) / 2
        
        return np.array([x, y]), confidence

## 7.2 Tracking Point Selection

### Option 1: Hip Center (Recommended)

The midpoint between left and right hip landmarks closely approximates barbell position during back squats because:

- The barbell rests on the upper back, moving in sync with the hips

- Hip landmarks are reliably detected by pose estimation models

- Less affected by arm movement than shoulder landmarks

### Option 2: Shoulder Center

The midpoint between shoulders can be used, but: - More variable due to potential forward lean changes - May be partially occluded by the barbell - Higher on the body than the actual barbell position

### Option 3: Direct Barbell Detection

For maximum accuracy, detect the barbell directly using: - Color-based segmentation (barbell sleeves are often distinct colors) - Circular Hough transform (detecting barbell plates) - YOLO or similar object detection trained on barbells

def detect_barbell_plates(frame):
    """
    Detect barbell plates using Hough Circle Transform.
    
    Returns estimated barbell center position.
    """
    gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)
    gray = cv2.GaussianBlur(gray, (9, 9), 2)
    
    circles = cv2.HoughCircles(
        gray,
        cv2.HOUGH_GRADIENT,
        dp=1,
        minDist=50,
        param1=100,
        param2=30,
        minRadius=30,
        maxRadius=150
    )
    
    if circles is not None:
        circles = np.uint16(np.around(circles))
        # Return center of detected circles
        centers = circles[0, :, :2]
        return np.mean(centers, axis=0)
    
    return None

## 7.3 Handling Missing Detections

When pose estimation fails for a frame:

class PositionTracker:
    def __init__(self, max_missing_frames=5):
        self.positions = []
        self.timestamps = []
        self.missing_count = 0
        self.max_missing = max_missing_frames
    
    def update(self, position, timestamp, confidence):
        """
        Update tracker with new position or handle missing detection.
        """
        if position is None or confidence < 0.3:
            self.missing_count += 1
            
            if self.missing_count <= self.max_missing and len(self.positions) >= 2:
                # Linear interpolation for short gaps
                interpolated = self._interpolate_position(timestamp)
                self.positions.append(interpolated)
                self.timestamps.append(timestamp)
            else:
                # Gap too large, reset tracking
                return False
        else:
            self.missing_count = 0
            self.positions.append(position)
            self.timestamps.append(timestamp)
        
        return True
    
    def _interpolate_position(self, timestamp):
        """Linear interpolation based on previous velocity."""
        if len(self.positions) < 2:
            return self.positions[-1]
        
        dt = self.timestamps[-1] - self.timestamps[-2]
        velocity = (self.positions[-1] - self.positions[-2]) / dt
        
        new_dt = timestamp - self.timestamps[-1]
        return self.positions[-1] + velocity * new_dt

# 8. Velocity Calculation Algorithms

## 8.1 Basic Velocity Calculation

Velocity is the rate of change of position:

v=ΔxΔt=xt-xt-1t-t-1

For vertical velocity during squats, we use only the y-component:

def calculate_instantaneous_velocity(positions, timestamps, pixels_per_meter):
    """
    Calculate instantaneous velocity for each frame.
    
    Args:
        positions: Array of (x, y) positions in pixels
        timestamps: Array of timestamps in seconds
        pixels_per_meter: Calibration factor
    
    Returns:
        velocities: Array of vertical velocities in m/s
    """
    velocities = []
    
    for i in range(1, len(positions)):
        # Vertical displacement in pixels (note: y increases downward in images)
        dy_pixels = positions[i-1][1] - positions[i][1]  # Invert for upward = positive
        
        # Convert to meters
        dy_meters = dy_pixels / pixels_per_meter
        
        # Time difference
        dt = timestamps[i] - timestamps[i-1]
        
        # Velocity
        v = dy_meters / dt
        velocities.append(v)
    
    return np.array(velocities)

## 8.2 Multi-Point Velocity Estimation

For better noise rejection, calculate velocity using multiple points:

def calculate_velocity_multipoint(positions, timestamps, pixels_per_meter, window=3):
    """
    Calculate velocity using linear regression over a window of points.
    
    This method is more robust to noise than simple differentiation.
    """
    velocities = []
    
    for i in range(window, len(positions)):
        # Extract window
        window_positions = positions[i-window:i+1]
        window_times = timestamps[i-window:i+1]
        
        # Convert to meters
        y_meters = (window_positions[:, 1] * -1) / pixels_per_meter  # Invert y
        
        # Linear regression: position = velocity * time + intercept
        # Using numpy's polyfit for least squares fit
        coeffs = np.polyfit(window_times - window_times[0], y_meters - y_meters[0], 1)
        velocity = coeffs[0]  # Slope is velocity
        
        velocities.append(velocity)
    
    return np.array(velocities)

## 8.3 Phase Detection

Identifying concentric and eccentric phases:

class RepPhaseDetector:
    def __init__(self, velocity_threshold=0.05):
        self.velocity_threshold = velocity_threshold
        self.current_phase = None
        self.phase_history = []
        
    def update(self, velocity):
        """
        Determine movement phase based on velocity.
        
        Phases:
        - 'eccentric': Downward movement (negative velocity)
        - 'concentric': Upward movement (positive velocity)
        - 'stationary': Minimal movement
        """
        if velocity > self.velocity_threshold:
            phase = 'concentric'
        elif velocity < -self.velocity_threshold:
            phase = 'eccentric'
        else:
            phase = 'stationary'
        
        # Detect phase transitions
        if self.current_phase != phase:
            if self.current_phase == 'eccentric' and phase == 'concentric':
                self.phase_history.append('bottom')
            elif self.current_phase == 'concentric' and phase in ['eccentric', 'stationary']:
                self.phase_history.append('top')
        
        self.current_phase = phase
        return phase
    
    def get_concentric_phases(self, velocities, timestamps):
        """
        Extract all concentric phase windows from velocity data.
        
        Returns list of (start_idx, end_idx) tuples.
        """
        phases = []
        in_concentric = False
        start_idx = None
        
        for i, v in enumerate(velocities):
            phase = self.update(v)
            
            if phase == 'concentric' and not in_concentric:
                in_concentric = True
                start_idx = i
            elif phase != 'concentric' and in_concentric:
                in_concentric = False
                phases.append((start_idx, i))
        
        # Handle case where recording ends during concentric
        if in_concentric:
            phases.append((start_idx, len(velocities)))
        
        return phases

## 8.4 Key Metrics Calculation

class VelocityMetrics:
    @staticmethod
    def mean_velocity(velocities, timestamps):
        """
        Calculate mean concentric velocity (MCV).
        
        MCV = total displacement / total time
        """
        total_displacement = np.trapz(velocities, timestamps)
        total_time = timestamps[-1] - timestamps[0]
        return total_displacement / total_time
    
    @staticmethod
    def peak_velocity(velocities):
        """
        Calculate peak concentric velocity (PCV).
        """
        return np.max(velocities)
    
    @staticmethod
    def velocity_at_percentage(velocities, percentage):
        """
        Get velocity at a specific percentage of the concentric phase.
        
        Args:
            velocities: Array of velocities during concentric
            percentage: 0-100 (e.g., 50 for mid-point)
        
        Returns:
            Velocity at that percentage of the lift
        """
        idx = int(len(velocities) * percentage / 100)
        idx = min(idx, len(velocities) - 1)
        return velocities[idx]
    
    @staticmethod
    def time_to_peak(velocities, timestamps):
        """
        Calculate time from start of concentric to peak velocity.
        """
        peak_idx = np.argmax(velocities)
        return timestamps[peak_idx] - timestamps[0]

# 9. Signal Processing and Filtering

## 9.1 Why Filtering is Necessary

Raw velocity data from frame-to-frame differentiation is noisy due to:

- Pose estimation jitter (landmark position varies slightly between frames)

- Camera sensor noise

- Quantization errors (limited pixel resolution)

- Motion blur during fast movements

## 9.2 Low-Pass Butterworth Filter

The most common filter for velocity data:

from scipy.signal import butter, filtfilt

def butterworth_lowpass_filter(data, cutoff_freq, sample_rate, order=2):
    """
    Apply zero-phase Butterworth low-pass filter.
    
    Args:
        data: Input signal (1D array)
        cutoff_freq: Cutoff frequency in Hz
        sample_rate: Sampling frequency in Hz
        order: Filter order (typically 2-4)
    
    Returns:
        Filtered signal
    """
    nyquist = sample_rate / 2
    normalized_cutoff = cutoff_freq / nyquist
    
    # Ensure cutoff is valid
    normalized_cutoff = min(normalized_cutoff, 0.99)
    
    b, a = butter(order, normalized_cutoff, btype='low')
    
    # filtfilt applies filter twice (forward and backward)
    # This achieves zero phase distortion
    filtered_data = filtfilt(b, a, data)
    
    return filtered_data

### Cutoff Frequency Selection

For barbell velocity during squats:

| Movement Phase | Typical Frequency Content | Recommended Cutoff |
| --- | --- | --- |
| Slow eccentric | 0.5-2 Hz | 3-5 Hz |
| Fast concentric | 1-4 Hz | 6-10 Hz |
| General purpose | 0.5-4 Hz | 6 Hz |

# Example usage
sample_rate = 60  # Hz (camera frame rate)
cutoff_freq = 6   # Hz

filtered_velocity = butterworth_lowpass_filter(
    raw_velocity, 
    cutoff_freq, 
    sample_rate, 
    order=2
)

## 9.3 Savitzky-Golay Filter

Alternative filter that preserves peaks better:

from scipy.signal import savgol_filter

def apply_savgol_filter(data, window_length=11, polyorder=2):
    """
    Apply Savitzky-Golay smoothing filter.
    
    This filter fits a polynomial to a window of points, which
    can better preserve peak velocities than simple averaging.
    
    Args:
        data: Input signal
        window_length: Number of points in window (must be odd)
        polyorder: Polynomial order (must be < window_length)
    
    Returns:
        Filtered signal
    """
    # Ensure window_length is odd
    if window_length % 2 == 0:
        window_length += 1
    
    return savgol_filter(data, window_length, polyorder)

## 9.4 Filter Comparison and Selection

def compare_filters(velocity, timestamps, sample_rate):
    """
    Compare different filtering approaches.
    """
    results = {}
    
    # Butterworth filters
    for cutoff in [4, 6, 8, 10]:
        key = f'butter_{cutoff}Hz'
        results[key] = butterworth_lowpass_filter(velocity, cutoff, sample_rate)
    
    # Savitzky-Golay filters
    for window in [5, 9, 13]:
        key = f'savgol_w{window}'
        results[key] = apply_savgol_filter(velocity, window_length=window)
    
    # Moving average
    for window in [3, 5, 7]:
        key = f'ma_w{window}'
        results[key] = np.convolve(velocity, np.ones(window)/window, mode='same')
    
    return results

def select_optimal_filter(filtered_results, reference_peak_velocity=None):
    """
    Select best filter based on noise reduction while preserving peaks.
    
    If reference_peak_velocity is provided (from gold standard),
    select filter that best matches the reference.
    """
    scores = {}
    
    for name, filtered in filtered_results.items():
        # Noise metric: standard deviation of velocity differences
        noise = np.std(np.diff(filtered))
        
        # Peak preservation: how close is peak to reference
        peak = np.max(filtered)
        
        if reference_peak_velocity:
            peak_error = abs(peak - reference_peak_velocity)
            scores[name] = {'noise': noise, 'peak_error': peak_error}
        else:
            scores[name] = {'noise': noise, 'peak': peak}
    
    return scores

## 9.5 Real-Time vs. Post-Processing Considerations

Real-time filtering (causal filters): - Only uses past and current data points - Introduces some phase lag - Use single-pass Butterworth: lfilter() instead of filtfilt()

Post-processing (non-causal filters): - Can use future data points - Zero phase distortion - Use filtfilt() for Butterworth

from scipy.signal import lfilter

def realtime_filter(data, cutoff_freq, sample_rate, order=2):
    """
    Causal filter for real-time applications.
    Note: This introduces phase lag.
    """
    nyquist = sample_rate / 2
    normalized_cutoff = cutoff_freq / nyquist
    b, a = butter(order, normalized_cutoff, btype='low')
    
    # Single-pass filter (causal)
    return lfilter(b, a, data)

# 10. Real-Time Implementation

## 10.1 System Architecture for Real-Time

import threading
import queue
import cv2
import time

class RealTimeVelocityTracker:
    def __init__(self, camera_id=0, fps_target=60, pixels_per_meter=500):
        # Camera setup
        self.cap = cv2.VideoCapture(camera_id)
        self.cap.set(cv2.CAP_PROP_FPS, fps_target)
        self.cap.set(cv2.CAP_PROP_FRAME_WIDTH, 1920)
        self.cap.set(cv2.CAP_PROP_FRAME_HEIGHT, 1080)
        
        # Processing components
        self.pose_extractor = PoseExtractor(model_complexity=1)
        self.position_tracker = PositionTracker()
        self.phase_detector = RepPhaseDetector()
        
        # Calibration
        self.pixels_per_meter = pixels_per_meter
        
        # Threading
        self.frame_queue = queue.Queue(maxsize=10)
        self.result_queue = queue.Queue(maxsize=10)
        self.running = False
        
        # Data storage
        self.positions = []
        self.timestamps = []
        self.velocities = []
        
        # Real-time filter state
        self._init_realtime_filter()
    
    def _init_realtime_filter(self, cutoff=6, sample_rate=60):
        """Initialize filter coefficients for real-time filtering."""
        from scipy.signal import butter
        nyquist = sample_rate / 2
        normalized_cutoff = cutoff / nyquist
        self.b, self.a = butter(2, normalized_cutoff, btype='low')
        self.filter_state = np.zeros(max(len(self.a), len(self.b)) - 1)
    
    def _realtime_filter_sample(self, sample):
        """Process single sample through filter with state."""
        from scipy.signal import lfilter_zi, lfilter
        filtered, self.filter_state = lfilter(
            self.b, self.a, [sample], zi=self.filter_state
        )
        return filtered[0]
    
    def capture_thread(self):
        """Thread for continuous frame capture."""
        while self.running:
            ret, frame = self.cap.read()
            if ret:
                timestamp = time.perf_counter()
                try:
                    self.frame_queue.put((frame, timestamp), timeout=0.01)
                except queue.Full:
                    pass  # Drop frame if queue is full
    
    def process_thread(self):
        """Thread for pose estimation and velocity calculation."""
        while self.running:
            try:
                frame, timestamp = self.frame_queue.get(timeout=0.1)
            except queue.Empty:
                continue
            
            # Extract pose
            h, w = frame.shape[:2]
            landmarks, confidence = self.pose_extractor.process_frame(frame)
            
            if landmarks is not None and confidence > 0.3:
                position, _ = self.pose_extractor.get_hip_center(landmarks, w, h)
                
                # Track position
                self.positions.append(position)
                self.timestamps.append(timestamp)
                
                # Calculate velocity
                if len(self.positions) >= 2:
                    dy_pixels = self.positions[-2][1] - self.positions[-1][1]
                    dy_meters = dy_pixels / self.pixels_per_meter
                    dt = self.timestamps[-1] - self.timestamps[-2]
                    raw_velocity = dy_meters / dt
                    
                    # Apply real-time filter
                    filtered_velocity = self._realtime_filter_sample(raw_velocity)
                    self.velocities.append(filtered_velocity)
                    
                    # Detect phase
                    phase = self.phase_detector.update(filtered_velocity)
                    
                    # Queue result
                    result = {
                        'timestamp': timestamp,
                        'position': position,
                        'velocity': filtered_velocity,
                        'phase': phase,
                        'frame': frame
                    }
                    
                    try:
                        self.result_queue.put(result, timeout=0.01)
                    except queue.Full:
                        pass
    
    def start(self):
        """Start capture and processing threads."""
        self.running = True
        self.capture_thread_handle = threading.Thread(target=self.capture_thread)
        self.process_thread_handle = threading.Thread(target=self.process_thread)
        self.capture_thread_handle.start()
        self.process_thread_handle.start()
    
    def stop(self):
        """Stop all threads and cleanup."""
        self.running = False
        self.capture_thread_handle.join()
        self.process_thread_handle.join()
        self.cap.release()
    
    def get_latest_result(self):
        """Get most recent processing result."""
        try:
            return self.result_queue.get_nowait()
        except queue.Empty:
            return None

## 10.2 Display and Feedback

def create_velocity_display(frame, velocity, phase, rep_count, target_velocity=None):
    """
    Create visual overlay for real-time feedback.
    """
    h, w = frame.shape[:2]
    
    # Velocity text
    vel_text = f"Velocity: {velocity:.2f} m/s"
    cv2.putText(frame, vel_text, (20, 50), 
                cv2.FONT_HERSHEY_SIMPLEX, 1.5, (0, 255, 0), 3)
    
    # Phase indicator
    phase_color = {
        'concentric': (0, 255, 0),   # Green
        'eccentric': (0, 0, 255),     # Red
        'stationary': (255, 255, 0)   # Cyan
    }.get(phase, (255, 255, 255))
    
    cv2.putText(frame, phase.upper(), (20, 100),
                cv2.FONT_HERSHEY_SIMPLEX, 1.2, phase_color, 3)
    
    # Rep counter
    cv2.putText(frame, f"Reps: {rep_count}", (20, 150),
                cv2.FONT_HERSHEY_SIMPLEX, 1.2, (255, 255, 255), 2)
    
    # Velocity bar
    bar_width = 50
    bar_height = int(min(abs(velocity) * 200, 400))  # Scale factor
    bar_x = w - 80
    bar_y = h // 2
    
    bar_color = (0, 255, 0) if velocity > 0 else (0, 0, 255)
    
    if velocity > 0:
        cv2.rectangle(frame, 
                      (bar_x, bar_y - bar_height),
                      (bar_x + bar_width, bar_y),
                      bar_color, -1)
    else:
        cv2.rectangle(frame,
                      (bar_x, bar_y),
                      (bar_x + bar_width, bar_y + bar_height),
                      bar_color, -1)
    
    # Target velocity indicator
    if target_velocity is not None:
        target_y = bar_y - int(target_velocity * 200)
        cv2.line(frame, (bar_x - 10, target_y), (bar_x + bar_width + 10, target_y),
                 (255, 255, 0), 3)
    
    return frame

## 10.3 Performance Optimization

def optimize_for_realtime():
    """
    Tips for achieving real-time performance.
    """
    optimizations = {
        'reduce_resolution': {
            'description': 'Process at lower resolution, then scale results',
            'implementation': 'cv2.resize(frame, (640, 480))'
        },
        'skip_frames': {
            'description': 'Process every Nth frame for slow hardware',
            'implementation': 'if frame_count % 2 == 0: process(frame)'
        },
        'roi_cropping': {
            'description': 'Only process region containing the lifter',
            'implementation': 'frame = frame[y1:y2, x1:x2]'
        },
        'gpu_acceleration': {
            'description': 'Use GPU for pose estimation',
            'implementation': 'Set CUDA as backend for OpenCV/MediaPipe'
        },
        'model_complexity': {
            'description': 'Use lighter pose estimation model',
            'implementation': 'model_complexity=0 in MediaPipe'
        }
    }
    
    return optimizations

# 11. Validation and Accuracy Assessment

## 11.1 Validation Against Gold Standard

To validate your system, compare against a reference device:

| Device | Accuracy | Cost | Availability |
| --- | --- | --- | --- |
| GymAware | ±0.02 m/s | $2000+ | Research labs |
| Linear Position Transducer | ±0.01 m/s | $500+ | Research labs |
| PUSH Band 2.0 | ±0.05 m/s | $250 | Consumer |
| Tendo Unit | ±0.02 m/s | $1500 | Gyms |

## 11.2 Validation Protocol

def validation_protocol(camera_system, reference_device, n_reps=50):
    """
    Protocol for validating camera-based system against reference.
    
    Procedure:
    1. Synchronize start of both systems
    2. Perform n_reps across multiple loads
    3. Calculate error metrics
    
    Recommended loads: 40%, 55%, 70%, 85%, 100% 1RM
    Reps per load: 10 (2-3 sets)
    """
    results = {
        'load_kg': [],
        'camera_mcv': [],
        'reference_mcv': [],
        'camera_pcv': [],
        'reference_pcv': []
    }
    
    # ... collect data ...
    
    return results

def calculate_validation_metrics(camera_values, reference_values):
    """
    Calculate error metrics for validation.
    """
    from scipy import stats
    
    errors = np.array(camera_values) - np.array(reference_values)
    
    metrics = {
        # Mean Absolute Error
        'MAE': np.mean(np.abs(errors)),
        
        # Root Mean Square Error
        'RMSE': np.sqrt(np.mean(errors**2)),
        
        # Systematic bias (mean error)
        'Bias': np.mean(errors),
        
        # Standard deviation of errors
        'SD': np.std(errors),
        
        # Limits of Agreement (Bland-Altman)
        'LoA_lower': np.mean(errors) - 1.96 * np.std(errors),
        'LoA_upper': np.mean(errors) + 1.96 * np.std(errors),
        
        # Correlation
        'r': stats.pearsonr(camera_values, reference_values)[0],
        
        # Typical Error of Estimate
        'TEE': np.std(errors) / np.sqrt(2),
        
        # Coefficient of Variation
        'CV': (np.std(errors) / np.mean(reference_values)) * 100
    }
    
    return metrics

## 11.3 Bland-Altman Analysis

import matplotlib.pyplot as plt

def bland_altman_plot(camera_values, reference_values, title="Bland-Altman Plot"):
    """
    Create Bland-Altman plot for method comparison.
    """
    mean_values = (np.array(camera_values) + np.array(reference_values)) / 2
    differences = np.array(camera_values) - np.array(reference_values)
    
    mean_diff = np.mean(differences)
    std_diff = np.std(differences)
    
    fig, ax = plt.subplots(figsize=(10, 6))
    
    ax.scatter(mean_values, differences, alpha=0.6)
    
    # Mean line
    ax.axhline(mean_diff, color='red', linestyle='-', 
               label=f'Mean: {mean_diff:.3f} m/s')
    
    # Limits of agreement
    ax.axhline(mean_diff + 1.96*std_diff, color='red', linestyle='--',
               label=f'+1.96 SD: {mean_diff + 1.96*std_diff:.3f} m/s')
    ax.axhline(mean_diff - 1.96*std_diff, color='red', linestyle='--',
               label=f'-1.96 SD: {mean_diff - 1.96*std_diff:.3f} m/s')
    
    ax.set_xlabel('Mean of Methods (m/s)')
    ax.set_ylabel('Difference (Camera - Reference) (m/s)')
    ax.set_title(title)
    ax.legend()
    
    plt.tight_layout()
    return fig

## 11.4 Expected Accuracy

Based on published research, a well-implemented camera-based HPE system can achieve:

| Metric | Expected Range | Notes |
| --- | --- | --- |
| Mean Absolute Error | 0.02-0.05 m/s | For MCV |
| Correlation (r) | 0.90-0.98 | High correlation expected |
| CV% | 3-8% | Coefficient of variation |
| Limits of Agreement | ±0.08-0.12 m/s | 95% of differences |

# 12. Practical Considerations and Limitations

## 12.1 Environmental Factors

### Lighting

- Best: Diffuse, even lighting from multiple sources

- Avoid: Strong backlighting, direct sunlight, flickering lights

- Minimum: 300 lux on the lifter

### Background

- Best: Solid, contrasting color (not same as clothing)

- Avoid: Busy patterns, moving objects, mirrors reflecting the lifter

- Note: Some pose models handle complex backgrounds better than others

### Clothing

- Best: Form-fitting clothes in contrasting colors

- Avoid: Baggy clothing that obscures joint positions, patterns that interfere with detection

## 12.2 Movement Considerations

### Occlusion

The barbell, spotter, or lifter’s own body can occlude joints:

- Back squat: Barbell may partially occlude shoulders

- Solution: Use hip position as primary tracking point

### Lateral Movement

Side-to-side movement during the lift can affect measurements:

- Issue: Not captured in 2D sagittal view

- Impact: Overestimates true vertical velocity

- Mitigation: Use 3D pose estimation or multiple cameras

### Bar Whip

At heavy loads, the barbell can oscillate:

- Issue: Joints move differently than bar ends

- Impact: Up to 0.02-0.03 m/s error at very heavy loads

- Mitigation: Use barbell detection in addition to HPE

## 12.3 System Limitations

| Limitation | Impact | Workaround |
| --- | --- | --- |
| Single camera, 2D tracking | Underestimates if movement has lateral component | Use multiple cameras or assume minimal lateral |
| Pose estimation latency | Delays real-time feedback | Use faster models, accept slight lag |
| Frame rate limits velocity resolution | At 30 FPS, ±0.03 m/s resolution limit | Use 60+ FPS camera |
| Calibration drift | Accuracy degrades if camera moves | Re-calibrate periodically |
| Varying light conditions | Detection confidence drops | Control lighting, use robust models |

## 12.4 When NOT to Use Camera-Based Tracking

- Competition settings: Use validated, approved devices

- Research requiring <0.02 m/s accuracy: Use linear position transducers

- Uncontrolled environments: Varying lighting, crowded spaces

- Maximum velocity movements: Olympic lifts may need higher frame rates

# 13. Sample Implementation Code

## 13.1 Complete Basic Implementation

"""
complete_vbt_system.py

A complete implementation of camera-based velocity tracking for barbell squats.
"""

import cv2
import numpy as np
import mediapipe as mp
from scipy.signal import butter, filtfilt
import time
from dataclasses import dataclass
from typing import List, Tuple, Optional

@dataclass
class RepData:
    """Data for a single rep."""
    mean_velocity: float
    peak_velocity: float
    duration: float
    timestamps: np.ndarray
    velocities: np.ndarray

class SquatVelocityTracker:
    def __init__(
        self,
        camera_id: int = 0,
        pixels_per_meter: float = 500.0,
        target_fps: int = 60,
        filter_cutoff: float = 6.0
    ):
        # Initialize camera
        self.cap = cv2.VideoCapture(camera_id)
        self.cap.set(cv2.CAP_PROP_FPS, target_fps)
        self.cap.set(cv2.CAP_PROP_FRAME_WIDTH, 1920)
        self.cap.set(cv2.CAP_PROP_FRAME_HEIGHT, 1080)
        
        # Initialize MediaPipe Pose
        self.mp_pose = mp.solutions.pose
        self.mp_draw = mp.solutions.drawing_utils
        self.pose = self.mp_pose.Pose(
            static_image_mode=False,
            model_complexity=1,
            smooth_landmarks=True,
            min_detection_confidence=0.5,
            min_tracking_confidence=0.5
        )
        
        # Calibration
        self.pixels_per_meter = pixels_per_meter
        
        # Filter setup
        self.filter_cutoff = filter_cutoff
        self.sample_rate = target_fps
        
        # Data storage
        self.positions: List[np.ndarray] = []
        self.timestamps: List[float] = []
        self.velocities: List[float] = []
        self.reps: List[RepData] = []
        
        # Phase tracking
        self.current_phase = 'stationary'
        self.rep_start_idx = None
        
    def calibrate(self, known_distance_meters: float = 2.2):
        """
        Interactive calibration using a known distance.
        
        Instructions:
        1. Click on two points of known distance (e.g., barbell ends)
        2. System calculates pixels per meter
        """
        print("Calibration Mode")
        print(f"Click two points that are {known_distance_meters}m apart")
        
        points = []
        
        def click_callback(event, x, y, flags, param):
            if event == cv2.EVENT_LBUTTONDOWN:
                points.append((x, y))
                print(f"Point {len(points)}: ({x}, {y})")
        
        cv2.namedWindow('Calibration')
        cv2.setMouseCallback('Calibration', click_callback)
        
        ret, frame = self.cap.read()
        while len(points) < 2:
            display = frame.copy()
            for p in points:
                cv2.circle(display, p, 5, (0, 255, 0), -1)
            cv2.imshow('Calibration', display)
            cv2.waitKey(1)
        
        cv2.destroyWindow('Calibration')
        
        # Calculate scale
        pixel_dist = np.sqrt(
            (points[1][0] - points[0][0])**2 + 
            (points[1][1] - points[0][1])**2
        )
        self.pixels_per_meter = pixel_dist / known_distance_meters
        print(f"Calibration complete: {self.pixels_per_meter:.1f} pixels/meter")
    
    def _get_hip_center(self, landmarks, frame_shape) -> Tuple[np.ndarray, float]:
        """Extract hip center position from landmarks."""
        h, w = frame_shape[:2]
        
        left_hip = landmarks[self.mp_pose.PoseLandmark.LEFT_HIP.value]
        right_hip = landmarks[self.mp_pose.PoseLandmark.RIGHT_HIP.value]
        
        x = (left_hip.x + right_hip.x) / 2 * w
        y = (left_hip.y + right_hip.y) / 2 * h
        confidence = (left_hip.visibility + right_hip.visibility) / 2
        
        return np.array([x, y]), confidence
    
    def _calculate_velocity(self) -> Optional[float]:
        """Calculate instantaneous velocity from recent positions."""
        if len(self.positions) < 2:
            return None
        
        # Vertical displacement (invert y: up is positive)
        dy_pixels = self.positions[-2][1] - self.positions[-1][1]
        dy_meters = dy_pixels / self.pixels_per_meter
        
        dt = self.timestamps[-1] - self.timestamps[-2]
        if dt <= 0:
            return None
        
        return dy_meters / dt
    
    def _filter_velocities(self, velocities: np.ndarray) -> np.ndarray:
        """Apply Butterworth low-pass filter to velocities."""
        if len(velocities) < 10:
            return velocities
        
        nyquist = self.sample_rate / 2
        normalized_cutoff = min(self.filter_cutoff / nyquist, 0.99)
        b, a = butter(2, normalized_cutoff, btype='low')
        
        return filtfilt(b, a, velocities)
    
    def _detect_phase(self, velocity: float) -> str:
        """Detect movement phase based on velocity."""
        threshold = 0.05  # m/s
        
        if velocity > threshold:
            return 'concentric'
        elif velocity < -threshold:
            return 'eccentric'
        else:
            return 'stationary'
    
    def _process_rep_completion(self, end_idx: int):
        """Process completed rep and calculate metrics."""
        if self.rep_start_idx is None:
            return
        
        start = self.rep_start_idx
        end = end_idx
        
        if end - start < 5:  # Minimum frames for valid rep
            return
        
        timestamps = np.array(self.timestamps[start:end])
        velocities = np.array(self.velocities[start:end])
        
        # Filter for final metrics
        filtered = self._filter_velocities(velocities)
        
        rep = RepData(
            mean_velocity=np.mean(filtered),
            peak_velocity=np.max(filtered),
            duration=timestamps[-1] - timestamps[0],
            timestamps=timestamps - timestamps[0],
            velocities=filtered
        )
        
        self.reps.append(rep)
        print(f"Rep {len(self.reps)}: MCV={rep.mean_velocity:.2f} m/s, "
              f"PCV={rep.peak_velocity:.2f} m/s")
    
    def process_frame(self, frame: np.ndarray, timestamp: float) -> dict:
        """
        Process a single frame and return results.
        
        Returns:
            dict with keys: position, velocity, phase, landmarks
        """
        # Convert to RGB for MediaPipe
        rgb = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)
        results = self.pose.process(rgb)
        
        output = {
            'position': None,
            'velocity': None,
            'phase': self.current_phase,
            'landmarks': None,
            'confidence': 0.0
        }
        
        if results.pose_landmarks:
            output['landmarks'] = results.pose_landmarks
            
            position, confidence = self._get_hip_center(
                results.pose_landmarks.landmark, 
                frame.shape
            )
            
            if confidence > 0.3:
                output['position'] = position
                output['confidence'] = confidence
                
                self.positions.append(position)
                self.timestamps.append(timestamp)
                
                velocity = self._calculate_velocity()
                if velocity is not None:
                    self.velocities.append(velocity)
                    output['velocity'] = velocity
                    
                    # Phase detection
                    new_phase = self._detect_phase(velocity)
                    
                    # Detect phase transitions
                    if self.current_phase == 'eccentric' and new_phase == 'concentric':
                        self.rep_start_idx = len(self.velocities) - 1
                    elif self.current_phase == 'concentric' and new_phase != 'concentric':
                        self._process_rep_completion(len(self.velocities))
                        self.rep_start_idx = None
                    
                    self.current_phase = new_phase
                    output['phase'] = new_phase
        
        return output
    
    def draw_overlay(self, frame: np.ndarray, results: dict) -> np.ndarray:
        """Draw visualization overlay on frame."""
        h, w = frame.shape[:2]
        
        # Draw pose landmarks
        if results['landmarks']:
            self.mp_draw.draw_landmarks(
                frame, results['landmarks'], self.mp_pose.POSE_CONNECTIONS
            )
        
        # Velocity display
        if results['velocity'] is not None:
            vel = results['velocity']
            color = (0, 255, 0) if vel > 0 else (0, 0, 255)
            text = f"Velocity: {vel:.2f} m/s"
            cv2.putText(frame, text, (20, 50), 
                        cv2.FONT_HERSHEY_SIMPLEX, 1.5, color, 3)
        
        # Phase display
        phase_colors = {
            'concentric': (0, 255, 0),
            'eccentric': (0, 0, 255),
            'stationary': (255, 255, 0)
        }
        cv2.putText(frame, results['phase'].upper(), (20, 100),
                    cv2.FONT_HERSHEY_SIMPLEX, 1.2, 
                    phase_colors.get(results['phase'], (255, 255, 255)), 3)
        
        # Rep counter
        cv2.putText(frame, f"Reps: {len(self.reps)}", (20, 150),
                    cv2.FONT_HERSHEY_SIMPLEX, 1.2, (255, 255, 255), 2)
        
        # Last rep metrics
        if self.reps:
            last_rep = self.reps[-1]
            cv2.putText(frame, f"Last MCV: {last_rep.mean_velocity:.2f} m/s", 
                        (20, h - 60),
                        cv2.FONT_HERSHEY_SIMPLEX, 1.0, (255, 255, 255), 2)
            cv2.putText(frame, f"Last PCV: {last_rep.peak_velocity:.2f} m/s", 
                        (20, h - 20),
                        cv2.FONT_HERSHEY_SIMPLEX, 1.0, (255, 255, 255), 2)
        
        return frame
    
    def run(self):
        """Main loop for real-time tracking."""
        print("Starting velocity tracking...")
        print("Press 'c' to calibrate, 'q' to quit")
        
        while True:
            ret, frame = self.cap.read()
            if not ret:
                break
            
            timestamp = time.perf_counter()
            
            # Process frame
            results = self.process_frame(frame, timestamp)
            
            # Draw overlay
            display = self.draw_overlay(frame.copy(), results)
            
            # Show frame
            cv2.imshow('Squat Velocity Tracker', display)
            
            key = cv2.waitKey(1) & 0xFF
            if key == ord('q'):
                break
            elif key == ord('c'):
                self.calibrate()
        
        self.cleanup()
    
    def cleanup(self):
        """Release resources."""
        self.cap.release()
        cv2.destroyAllWindows()
        self.pose.close()
    
    def get_session_summary(self) -> dict:
        """Get summary statistics for the session."""
        if not self.reps:
            return {}
        
        mcvs = [r.mean_velocity for r in self.reps]
        pcvs = [r.peak_velocity for r in self.reps]
        
        return {
            'total_reps': len(self.reps),
            'mean_mcv': np.mean(mcvs),
            'std_mcv': np.std(mcvs),
            'mean_pcv': np.mean(pcvs),
            'std_pcv': np.std(pcvs),
            'velocity_loss': (mcvs[0] - mcvs[-1]) / mcvs[0] * 100 if len(mcvs) > 1 else 0
        }


if __name__ == '__main__':
    tracker = SquatVelocityTracker(
        camera_id=0,
        pixels_per_meter=500,  # Will be updated by calibration
        target_fps=60,
        filter_cutoff=6
    )
    tracker.run()
    
    # Print session summary
    summary = tracker.get_session_summary()
    print("\nSession Summary:")
    for key, value in summary.items():
        print(f"  {key}: {value:.2f}" if isinstance(value, float) else f"  {key}: {value}")

# 14. Troubleshooting Guide

## 14.1 Common Issues and Solutions

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| No pose detected | Poor lighting | Improve lighting, avoid backlighting |
| Pose detected but jittery | Low confidence | Increase detection threshold, improve contrast |
| Velocity seems wrong | Calibration error | Re-calibrate with accurate measurement |
| Inconsistent frame rate | Camera/CPU limitation | Reduce resolution, use lighter model |
| Phase detection unreliable | Threshold too high/low | Adjust velocity threshold |
| Missed rep counts | Movement too fast | Increase frame rate, adjust thresholds |
| High noise in velocity | Filter cutoff too high | Lower filter cutoff frequency |
| Velocities clipped | Filter cutoff too low | Raise filter cutoff frequency |

## 14.2 Diagnostic Checks

def run_diagnostics(tracker):
    """Run diagnostic checks on the tracking system."""
    print("Running diagnostics...")
    
    # Check camera
    ret, frame = tracker.cap.read()
    if not ret:
        print("ERROR: Cannot read from camera")
        return False
    
    h, w = frame.shape[:2]
    print(f"Camera resolution: {w}x{h}")
    
    # Check frame rate
    timestamps = []
    for _ in range(60):
        ret, _ = tracker.cap.read()
        timestamps.append(time.perf_counter())
    
    actual_fps = (len(timestamps) - 1) / (timestamps[-1] - timestamps[0])
    print(f"Actual FPS: {actual_fps:.1f}")
    
    # Check pose detection
    ret, frame = tracker.cap.read()
    rgb = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)
    results = tracker.pose.process(rgb)
    
    if results.pose_landmarks:
        print("Pose detection: OK")
        
        # Check key landmark visibility
        landmarks = results.pose_landmarks.landmark
        key_points = ['LEFT_HIP', 'RIGHT_HIP', 'LEFT_SHOULDER', 'RIGHT_SHOULDER']
        for name in key_points:
            idx = getattr(tracker.mp_pose.PoseLandmark, name).value
            visibility = landmarks[idx].visibility
            status = "OK" if visibility > 0.5 else "LOW"
            print(f"  {name}: {visibility:.2f} ({status})")
    else:
        print("Pose detection: FAILED - no pose found")
        return False
    
    # Check calibration
    print(f"Calibration: {tracker.pixels_per_meter:.1f} pixels/meter")
    if tracker.pixels_per_meter < 100 or tracker.pixels_per_meter > 2000:
        print("WARNING: Calibration value seems unusual")
    
    print("Diagnostics complete")
    return True

# 15. References and Further Reading

## 15.1 Academic References

- Bazuelo-Ruiz, B., et al. (2018). “Validity of linear and inertial measurement units for measuring velocity in the back squat.” Journal of Sports Sciences, 36(13), 1549-1555.

- Courel-Ibáñez, J., et al. (2019). “Reliability and validity of a single camera-based markerless motion capture system to determine velocity during the back squat.” Sports Biomechanics, 18(4), 464-477.

- Orange, S.T., et al. (2020). “Validity and reliability of a simple smartphone-based application for velocity-based resistance training.” Journal of Sports Sciences, 38(10), 1090-1097.

- Sayers, M.G.L., et al. (2018). “The validity of a smartphone application compared to criterion measures for kinetic and kinematic variables during a back squat exercise.” International Journal of Exercise Science, 11(7), 1007-1018.

## 15.2 Technical Documentation

- MediaPipe Pose: https://google.github.io/mediapipe/solutions/pose

- OpenCV Documentation: https://docs.opencv.org/

- SciPy Signal Processing: https://docs.scipy.org/doc/scipy/reference/signal.html

## 15.3 VBT Resources

- Science for Sport VBT Guide: https://www.scienceforsport.com/velocity-based-training/

- MASS Research Review VBT Articles

- Stronger by Science VBT Analysis

# Appendix A: Quick Start Checklist

- ☐ Camera capable of 60+ FPS at 1080p

- ☐ Computer with adequate processing power

- ☐ Python 3.8+ installed

- ☐ Required libraries installed (opencv, mediapipe, scipy, numpy)

- ☐ Camera positioned perpendicular to lifter, 3-5m away

- ☐ Adequate lighting with no backlighting

- ☐ Known distance object for calibration (e.g., barbell)

- ☐ Test run with pose detection verified

- ☐ Calibration completed

- ☐ Filter parameters tuned for your frame rate

# Appendix B: Glossary

| Term | Definition |
| --- | --- |
| MCV | Mean Concentric Velocity - average velocity during the lifting phase |
| PCV | Peak Concentric Velocity - maximum velocity achieved during the lift |
| VBT | Velocity-Based Training - training methodology using movement velocity |
| HPE | Human Pose Estimation - computer vision technique to detect body keypoints |
| FPS | Frames Per Second - camera capture rate |
| LPT | Linear Position Transducer - gold standard velocity measurement device |
| Concentric | The lifting/shortening phase of an exercise |
| Eccentric | The lowering/lengthening phase of an exercise |
| Load-Velocity Profile | Relationship between weight lifted and movement velocity |
| Cutoff Frequency | Frequency above which the filter attenuates signals |

Document Version: 1.0 Last Updated: March 2026